3D- BUILDER MODULE

- Click Tools and then Setup and follow the instructions.

On the right panel, click on level 0 and select modify height and enter 0.5. Press ok

- Click rectangle and select Tools  Keyboard entry. Enter (-50,-50) and (50, 50).

- Similar to step 2 click on level 1 and modify height and enter 0.5.

- Modify the entity and select entity 4.

- Now click rectangle and follow the instructions. Two coordinates are (-20,-50)and (-10,-47.5)

- Now click rectangle and follow the instructions. Two coordinates are (-10,-45) and (-47.5,-47.5).

- Click rectangle and follow the instructions. Two coordinates are (-47.5,-47.5) and (-45,47.5)

- Select the last 3 rectangles drawn and click on mirror and then click Tools Keyboard entry

You should get a figure as shown below.

- Click rectangle and give the two coordinates (-47.5, 47.5) and (47.5, 45).

- Click Mesh  Validate mesh connectivity. This checks for overlapped elements.

- You will have some misaligned elements.

- Similarly Click Mesh  Validate Mesh conformity. You will have no illegal elements.

- To solve the error at step 12,  select Mesh Auto mesh and give mesh size of 30.

- Now Click File and Export to Analysis Module. At that time, the builder will ask to check for mesh validity and now you will find no illegal or misaligned elements.