BLUE PRINT MODULE

- Click Snaps and select Grid

- Check if the grid values of X and Y are 10.

- Click on the rectangle tool bar and command window will ask for coordinates.

Two coordinates are (50, 50) and (-50,-50)

You can also select one coordinate manually and click and drag upto the second coordinate to construct a rectangle.

- Click on Layers and select New

- Click on the drop down and select Layer 1

- Click on the rectangle and follow the instructions. Two coordinates are (-20,-50)and (-10,-47.5)

- In order to repeat the previous step, i.e selecting a rectangle, you can just type enter and give the two coordinates (-10,-45) and (-47.5,-47.5).

- Click rectangle and follow the instructions. Two coordinates are (-47.5,-47.5) and (-45,47.5)

Click on mirror and select the last three rectangles drawn and the command window asks for two points of reflection.

First point: (0, 0) and press enter

Second point: (0, 10), press enter.

- Click rectangle and give the two coordinates (-47.5, 47.5) and (47.5, 45).

Select the rectangles in blue colour (all 7) and click path finder and Union. It will ask for the destination layer. Select Layer 1 (current layer).